Upload operating system, configuration file or GPS location Table over the air using GPRS:

The Operating System Version 7.1 or higher will allow you to update the complete operating system, configuration file or GPS locations Table over the air using GPRS.

Before you can update the operating system, configuration file or GPS locations Table over the air you need to complete the following setup.

1. An internet connection with Static IP address.

2. Port "6081" (or any other port you want to use to upload the operating system) needs to be routed from your Router or Firewall to a computer system running the Tracking System Remote Download Program.

3. You need to have configured the 'Static IP address for UDP' and the 'UDP OS upload port' with the correct settings for uploading.

4. Make sure that GPRS is enabled for the SIM card in your tracking unit.

Once you setup and configure the above settings you can send the upload command to the tracking unit for upload of the operating system: 05*827,14,0 or 05*827,15,0 to upload the configuration file.

There is no need to configure the tracking unit for 'SMS and GPRS' as the upload command will always use GPRS to upload the operating system, configuration file or GPS location Table even if your tracking unit is configured for SMS only.

After uploading the operating system or configuration file the tracking unit will verify the 'program checksum'. If correct then the system will automatically reboot. Operating system upload over the air will not alter any configuration settings (operating system upload using the serial port requires the reconfiguration of the configuration menu and settings). Configuration file updates over the air will also automatic reboot the system to enable the tracking unit with the new settings.

GPS location Table update will not reboot the tracking system.

If there are any errors during download then you will need to resend the upload command(s) again.

Tracking System Operating System Download Program:

This program can be used to update the operating system, update the configuration file or update the GPS locations Table for the tracking unit over the air.

At present the current version 1.2 can only download data to one unit at a time.

Set the download port for the tracking unit (default 6081). Press the 'Set host/port' to select the current port.

The 'Select Drive' option will select where the new operating system to download to the tracking unit is located.

As there may be several different tracking unit versions you will need to create a directory where the operating system for download is located.

As an example the unit you have is version V007 and your 'Select Drive' is C:.

Then you need to create a directory called: V007  (C:\V007) and copy the operating system you want to download to your unit (one (1) file with extension .s19) to this directory.

The 'Start' button will start the program scanning the selected port for any UDP data.

For configuration file update select the configuration file to download to your tracking system.

Tracking unit download sequence:

The tracking unit will send the following command to try to connect to the 'Tracking System Operating System Download Program':

<UDP packet identification>,<current version number>,UPLOAD

(Example: trace1,V007,UPLOAD).

The 'Tracking System Remote Download Program' will 'scan' the 'V007' directory for the first file with extension .s19.

This file will be used to download to the tracking unit.

The 'Tracking System Operating System Download Program' sends data in small blocks to the tracking unit and the tracking unit will confirm each packet after correct download only.

If any errors are detected the tracking unit will request the incorrect data block to be sent again.

The last data block sent will have the packet number: 255 (last packet).

After this the tracking unit will disconnect and update the operating system.

After download the tracking unit will automatically reset and restart the new uploaded operating system.

The tracking unit will send the following commands to try to connect to the 'Tracking System Remote Download Program' for configuration file or GPS location Table update:

<UDP packet identification>,<current version number>,CONFIG

(Example: trace1,V007,CONFIG).

<UDP packet identification>,<current version number>,GPSLOC

(Example: trace1,V007,GPSLOC).

Configuration file update will send the selected configuration file to your tracking unit.

GPS location Table update will send the selected GPS location Table file to your tracking unit.

Updated 11-8-2004